ENERGY, POWER, WORK CONVERSION FACTORS

| Energy Conversion Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Btu | foot-pound | joule | calorie | Kilowatt-hour |
| Btu | 1 | 779 | 1056 | 252 | 2.93x10-4 |
| foot-pound | 0.00129 | 1 | 1.36 | 0.324 | 3.77x10-7 |
| joule | 9.48x10-4 | 0.738 | 1 | .239 | 2.78x10-7 |
| calorie | 0.00397 | 3.09 | 4.19 | 1 | 1.16x10-6 |
| Kilowatt-hour | 3413 | 2.65x106 | 3.6x106 | 8.60x105 | 1 |

ENERGY:  The ability to do Work  (Btu, foot-pound, joule, calorie, watt hour)

WORK:  Force x Displacement

POWER:  Work / Time (Btu/hr, MMBtu/hr, kilowatt)

| Power Conversion Table |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Boiler Horse-power | Lbs Steam per hour | Gallons Water per hour | Btu/hr | MMBtu/hr |
| Boiler Horsepower | 1 | 34.5 | 0.242 | 33,500 | 0.0335 |
| Lbs Steam per hour | 0.029 | 1 | 0.120 | 971 | 9.71x10-4 |
| Gallons Water per Hour | 4.14 | 8.33 | 1 | 8107 | 8.11x10-3 |
| Btu/hr | 2.99x10-5 | 1.03x10-3 | 1.23x10-4 | 1 | 1x10-6 |
| MMBtu/hr | 29.85 | 1030 | 123.4 | 1x106 | 1 |

1 watt = 3.41 Btu/hr		1 Megawatt = 3.41 MMBtu/hr  	1000 Btu/hr = 293 watts

| Average Energy Content of Various Fuels |  |  |
| --- | --- | --- |
| Quantity of Fuel | Equates to | Energy Content |
| 1 Cubic Foot of natural gas | = | 1,008 to 1,034 Btu |
| 1 Therm of natural gas | = | 100,000 Btu |
| 1 gallon of crude oil | = | 138,095 Btu |
| 1 gallon of residual fuel oil | = | 149,690 Btu |
| 1 gallon of gasoline | = | 125,000 Btu |
| 1 pound of coal 1 pound of wood | = = | 8,100 to 13,000 Btu 4,500 to 8,500 Btu |
| 1 ton wood | = | 9 to 17 MMBtu |
| 1 pound of low pressure steam | = | 1,000 Btu |